U2FHID protocol description

Implementation specification

Version: 0.5
Date: 21th July, 2014

Disclaimer

The contents of this document are subject to revision without notice due to continued progress in methodology, design, and manufacturing. Yubico shall have no liability for any error or damages of any kind resulting from the use of this document.

Trademarks

Yubico and YubiKey are trademarks of Yubico AB.

Contact Information

Yubico AB

Kungsgatan 37, 8 floor
111 56 Stockholm
Sweden
info@yubico.com

Contents

1	Document Information	4

1.1	Purpose	4

1.2	Audience	4

1.3	Related documentation	4

1.4	Document History	4

1.5	Definitions	4

2	U2FHID protocol implementation	5

2.1	U2FHID implementation rationale	5

2.2	Protocol structure and data framing	5

2.3	Concurrency and channels	5

2.4	Message- and packet structure	6

2.5	Arbitration	7

2.6	Channel locking	8

2.7	Protocol version and compatibility	8

3	HID device implementation	9

3.1	Interface- and endpoint descriptors	9

3.2	HID report descriptor and device discovery	10

4	U2FHID commands	11

4.1	Mandatory commands	11

4.2	Optional commands	13

4.3	Vendor specific commands	13

# Document Information

## Purpose

The purpose of this documentation is to provide a complete specification how to implement the U2FHID protocol, where FIDO U2F messages are framed for USB transport, using the HID protocol. General FIDO and U2F- concepts, semantics,  meaning is beyond the scope of this document and for information on these topics, please refer to the appropriate related documentation.

## Audience

This document is intended for U2F host- and device developers.

## Related documentation

- fido-glossary-v1.0-rd-20140209.pdf

- fido-security-ref-v1.0-rd-20140209.pdf

- fido-u2f-application-isolation-through-facet-identification-v1.0-rd-20140209.pdf

- fido-u2f-implementation-considerations-v1.0-rd-20140209.pdf

- fido-u2f-javascript-api-v1.0-rd-20140209.pdf

- fido-u2f-overview-v1.0-rd-20140209.pdf

- fido-u2f-raw-message-formats-v1.0-rd-20140209.pdf

- fido-u2f-usb-framing-of-apdus-v1.0-rd-20140209.pdf

## Document History

| Date | Version | Author | Activity |
| --- | --- | --- | --- |
| 2014-03-27 | 0.1 | JE | First draft |
| 2014-05-20 | 0.2 | JE | Second draft |
| 2014-06-18 | 0.3 | JE | Third draft |
| 2014-06-24 | 0.4 | JE | Minor changes |
| 2014-07-21 | 0.5 | JE | Updated INIT, removed SYNC |

## Definitions

| Term | Definition |
| --- | --- |
| U2F | Universal Second Factor |
| USB | Universal Serial Bus |
| HID | Human Interface Device. A specification of typical USB devices used for human interaction, such as keyboards, mice, joysticks etc. |
| U2FHID | U2F transport over HID as defined by this document |

# U2FHID protocol implementation

This description does not describe the actual U2F messages, semantics and functionality but rather how such messages are framed for HID transport.

## U2FHID implementation rationale

The U2FHID protocol is designed with the following design objectives in mind

- Driver-less installation on all major host platforms

- Multi-application support with concurrent application access without the need for serialization and centralized dispatching.

- Fixed latency response and low protocol overhead

- Scalable method for U2FHID device discovery

Since HID data is sent as interrupt packets and multiple applications may access the HID stack at once, a non-trivial level of complexity has to be added to handle this.

## Protocol structure and data framing

The U2F protocol is designed to be concurrent and state-less in such a way that each performed function is not dependent on previous actions. However, there has to be some form of "atomicity" that varies between the characteristics of the underlying transport protocol, which for the U2FHID protocol introduces the following terminology:

- Transaction

- Message

- Packet

A transaction is the highest level of aggregated functionality, which in turn consists of a request, followed by a response message. Once a request has been initiated, the transaction has to be entirely completed before a second transaction can take place and a response is never sent without a previous request.

Request- and response messages are in turn divided into individual fragments, known as packets. The packet is the smallest form of protocol data unit, which in the case of U2FHID are mapped into HID reports.

## Concurrency and channels

Additional logic and overhead is required to allow a U2FHID device to deal with multiple "clients", i.e. multiple applications accessing the single resource through the HID stack. Each client communicates with a U2FHID device through a logical channel, where each application uses a unique 32-bit channel identifier for routing- and arbitration purposes.

A channel identifier is allocated by the U2F device to ensure its system-wide uniqueness. The actual algorithm for generation of channel identifiers is vendor specific and not defined by this specification.

Channel ID 0 is reserved and 0xffffffff is reserved for broadcast commands, i.e. at the time of channel allocation.

## Message- and packet structure

Packets are one of two types, initialization packets and continuation packets. As the name suggests, the first packet sent in a message is an initialization packet, which also becomes the start of a transaction. If the entire message does not fit into one packet (including the U2FHID protocol overhead), one or more continuation packets have to be sent in strict ascending order to complete the message transfer.

A message sent from a host to a device is known as a request and a message sent from a device back to the host is known as a response. A request always triggers a response and response messages are never sent ad-hoc, i.e. without a prior request message.

The request and response messages have an identical structure. A transaction is started with the initialization packet of the request message and ends with the last packet of the response message.

Packets are always fixed size (defined by the endpoint- and HID report descriptors) and although all bytes may not be needed in a particular packet, the full size always has to be sent. Unused bytes should be set to zero.

An initialization packet is defined as

| Offset | Length | Mnemonic | Description |
| --- | --- | --- | --- |
| 0 | 4 | CID | Channel identifier |
| 4 | 1 | CMD | Command identifier (bit 7 always set) |
| 5 | 1 | BCNTH | High part of payload length |
| 6 | 1 | BCNTL | Low part of payload length |
| 7 | (s - 7) | DATA | Payload data (s is equal to the fixed packet size) |

The command byte has always the highest bit set to distinguish it from a continuation packet, which is described below.

A continuation packet is defined as

| Offset | Length | Mnemonic | Description |
| --- | --- | --- | --- |
| 0 | 4 | CID | Channel identifier |
| 4 | 1 | SEQ | Packet sequence 0x00..0x7f (bit 7 always cleared) |
| 5 | (s - 5) | DATA | Payload data (s is equal to the fixed packet size) |

With this approach, a message with a payload less or equal to (s - 7) may be sent as one packet. A larger message is then divided into one or more continuation packets, starting with sequence number 0, which then increments by one to a maximum of 127.

With a packet size of 64 bytes (max for full-speed devices), this means that the maximum message payload length is 64 - 7 + 128 * (64 - 5) = 7609 bytes.

## Arbitration

In order to handle multiple channels and clients concurrency, the U2FHID protocol has to maintain certain internal states, block conflicting requests and maintain protocol integrity. The protocol relies on each client application (channel) behaves politely, i.e. does not actively act to destroy for other channels. With this said, a malign- or malfunctioning application can cause issues for other channels. Expected errors and potentially stalling applications should however be handled properly.

### Transaction atomicity, idle- and busy states.

A transaction always consists of three stages:

- A message is sent from the host to the device

- The device processes the message

- A response is sent back from the device to the host

The protocol is built on the assumption that a plurality of concurrent applications may try ad-hoc to perform transactions at any time, with each transaction being atomic, i.e. it cannot be interrupted by another application once started.

The application channel that manages to get through the first initialization packet when the device is in idle state will keep the device locked for other channels until the last packet of the response message has been received. The device then returns to idle state, ready to perform another transaction for the same or a different channel. Between two transactions, no state is maintained in the device and a host application must assume that any other process may execute other transactions at any time.

If an application tries to access the device from a different channel while the device is busy with a transaction, that request will immediately fail with a busy-error message sent to the requesting channel.

### Transaction timeout

A transaction has to be completed within a specified period of time to prevent a stalling application to cause the device to be completely locked out for access by other applications. If for example an application sends an initialization packet that signals that continuation packets will follow and that application crashes, the device will back out that pending channel request and return to an idle state.

### Transaction abort and resynchronization

If an application for any reason "gets lost", gets an unexpected response or error, it may at any time issue an abort-and-resynchronize command. If the device detects a SYNC command during a transaction that has the same channel id as the active transaction, the transaction is aborted (if possible) and all buffered data flushed (if any). The device then returns to idle state to become ready for a new transaction.

### Packet sequencing

The device keeps track of packets arriving in correct and ascending order and that no expected packets are missing. The device will continue to assemble a message until all parts of it has been received or that the transaction times out. Spurious continuation packets appearing without a prior initialization packet will be ignored.

## Channel locking

In order to deal with aggregated transactions that may not be interrupted, such as vendor specific tunnelling of APDUs, a channel lock command may be implemented. By sending a channel lock command, the device prevents other channels from communicating with the device until the channel lock has timed out or been explicitly unlocked by the application.

This feature is optional and has not to be considered by general U2F HID applications.

## Protocol version and compatibility

The U2FHID protocol is designed to be extensible, yet maintaining backwards compatibility to the extent it is applicable. This means that a U2FHID host shall support any version of a device with the command set available in that particular version.

# HID device implementation

This description assumes knowledge of the USB- and HID specifications and is intended to provide the basics for implementing a U2FHID device. There are several ways to implement USB devices and reviewing these different methods is beyond the scope of this document. This specification targets the interface part, where a device is regarded as either a single- or multiple interface (composite) device.

The description further assumes (but is not limited to) a full-speed USB device (12 Mbit/s). Although not excluded per se, USB low-speed devices are not practical to use given the 8-byte report size limitation together with the protocol overhead.

## Interface- and endpoint descriptors

The device implements two endpoints (except the control endpoint 0), one for IN- and one for OUT transfers. The packet size is vendor defined, but the reference implementation assumes a full-speed device with two 64-bytes endpoints.

| Interface descriptor |  |  |
| --- | --- | --- |
| Mnemonic | Value | Description |
| bNumEndpoints | 2 | One IN- and one OUT endpoint |
| bInterfaceClass | 0x03 | HID |
| bInterfaceSubClass | 0x00 | No interface subclass |
| bInterfaceProtocol | 0x00 | No interface protocol |

| Endpoint 1 descriptor |  |  |
| --- | --- | --- |
| Mnemonic | Value | Description |
| bmAttributes | 0x03 | Interrupt transfer |
| bEndpointAdresss | 0x01 | 1, OUT |
| bMaxPacketSize | 64 | 64 bytes packets |
| bInterval | 5 | Poll every 5 millisecond |

| Endpoint 2 descriptor |  |  |
| --- | --- | --- |
| Mnemonic | Value | Description |
| bmAttributes | 0x03 | Interrupt transfer |
| bEndpointAdresss | 0x81 | 1, IN |
| bMaxPacketSize | 64 | 64 bytes packets |
| bInterval | 5 | Poll every 5 millisecond |

The actual endpoint order, intervals, endpoint numbers and endpoint packet size may be defined freely by the vendor and the host application is responsible for querying these values and handle these accordingly. For the sake of clarity, the values listed above are used in the following examples.

## HID report descriptor and device discovery

A HID report descriptor is required for all HID devices, even though the reports and their interpretation (scope, range, etc.) makes very little sense from an operating system perspective. The U2FHID just provides two "raw" reports, which basically map directly to the IN and OUT endpoints.  However, the HID report descriptor has an important purpose in U2FHID, as it is used for device discovery.

For the sake of clarity, a bit of high-level C-style abstraction is provided

// HID report descriptor

const uint8_t HID_ReportDescriptor[] = {

HID_UsagePage       ( FIDO_USAGE_PAGE                        ),

HID_Usage           ( FIDO_USAGE_U2FHID                      ),

HID_Collection      ( HID_Application                        ),

HID_Usage         ( FIDO_USAGE_DATA_IN                     ),

HID_LogicalMin    ( 0                                      ),

HID_LogicalMaxS   ( 0xff                                   ),

HID_ReportSize    ( 8                                      ),

HID_ReportCount   ( HID_INPUT_REPORT_BYTES                 ),

HID_Input         ( HID_Data | HID_Absolute | HID_Variable ),

HID_Usage         ( FIDO_USAGE_DATA_OUT                    ),

HID_LogicalMin    ( 0                                      ),

HID_LogicalMaxS   ( 0xff                                   ),

HID_ReportSize    ( 8                                      ),

HID_ReportCount   ( HID_OUTPUT_REPORT_BYTES                ),

HID_Output        ( HID_Data | HID_Absolute | HID_Variable ),

HID_EndCollection

};

A unique Usage Page is defined for the FIDO alliance and under this realm, a U2FHID Usage is defined as well. During U2FHID device discovery, all HID devices present in the system are examined and devices that match this usage pages and usage are then considered to be U2FHID devices.

The length values specified by the HID_INPUT_REPORT_BYTES and the HID_OUTPUT_REPORT_BYTES shoud typically match the respective endpoint sizes defined in the endpoint descriptors.

# U2FHID commands

The U2FHID protocol implements the following commands. These commands are not related to U2F commands, which are send using the U2FHID_MSG command

## Mandatory commands

The following list describes the minimum set of commands required by an U2FHID device. Optional- and vendor-specific commands may be implemented as described in respective sections of this document.

### U2FHID_MSG

This command sends an encapsulated U2F message to the device. The semantics of the data message is defined in the U2F protocol specification.

| Request |  |
| --- | --- |
| CMD | U2FHID_MSG |
| BCNT | 4..n |
| DATA | n bytes |

| Response at success |  |
| --- | --- |
| CMD | U2FHID_MSG |
| BCNT | 2..n |
| DATA | N bytes |

### U2FHID_INIT

This command synchronizes a channel and optionally requests the device to allocate a unique 32-bit channel identifier (CID) that can be used by the requesting application during its lifetime. The requesting application generates a nonce that is used to match the response. When the response is received, the application compares the sent nonce with the received one. After a positive match, the application stores the received channel id and uses that for subsequent transactions.

To allocate a new channel, the requesting application shall use the broadcast channel U2FHID_BROADCAST_CID. The device then responds the newly allocated channel in the response, using the broadcast channel.

| Request |  |
| --- | --- |
| CMD | U2FHID _INIT |
| BCNT | 8 |
| DATA | 8 byte nonce |

| Response at success |  |
| --- | --- |
| CMD | U2FHID _INIT |
| BCNT | 17 (see note below) |
| DATA | 8 byte nonce |
| DATA+8 | 4 byte channel ID |
| DATA+12 | U2FHID protocol version identifier |
| DATA+13 | Major device version number |
| DATA+14 | Minor device version number |
| DATA+15 | Build device version number |
| DATA+16 | Capabilities flags |

The protocol version identifies the protocol version implemented by the device. An U2FHID host shall accept a response size that is longer than the anticipated size to allow for future extensions of the protocol, yet maintaining backwards compatibility. Future versions will maintain the response structure to this current version, but additional fields may be added.

The meaning and interpretation of the version number is vendor defined.

The following device capabilities flags are defined. Unused values are reserved for future use and must be set to zero by device vendors.

| Capabilites flags |  |
| --- | --- |
| CAPABILITY_WINK | Implements the WINK function |

### U2FHID_PING

Sends a transaction to the device, which immediately echoes the same data back. This command is defined to be an uniform function for debugging-, latency- and performance measurements.

| Request |  |
| --- | --- |
| CMD | U2FHID_PING |
| BCNT | 0..n |
| DATA | n bytes |

| Response at success |  |
| --- | --- |
| CMD | U2FHID_PING |
| BCNT | n |
| DATA | N bytes |

### U2FHID_ERROR

This command code is used in response messages only.

| Response |  |
| --- | --- |
| CMD | U2FHID_ERROR |
| BCNT | 1 |
| DATA | Error code |

The following error codes are defined

| ERR_INVALID_CMD | The command in the request is invalid |
| --- | --- |
| ERR_INVALID_PAR | The parameter(s) in the request is invalid |
| ERR_INVALID_LEN | The length field (BCNT) is invalid for the request |
| ERR_INVALID_SEQ | The sequence does not match expected value |
| ERR_MSG_TIMEOUT | The message has timed out |
| ERR_CHANNEL_BUSY | The device is busy for the requesting channel |

## Optional commands

The following commands are defined by this specification but are optional and does not have to be implemented.

### U2FHID_WINK

The wink command performs a vendor-defined action that provides some visual- or audible identification a particular U2F device. A typical implementation will do a short burst of flashes with a LED or something similar. This is useful when more than one device is attached to a computer and there is confusion which device is paired with which connection.

| Request |  |
| --- | --- |
| CMD | U2FHID_WINK |
| BCNT | 0 |
| DATA | N/A |

| Response at success |  |
| --- | --- |
| CMD | U2FHID_WINK |
| BCNT | 0 |
| DATA | N/A |

### U2FHID_LOCK

The lock command places an exclusive lock for one channel to communicate with the device. As long as the lock is active, any other channel trying to send a message will fail. In order to prevent a stalling- or crashing application to lock the device indefinitely, a lock time up to 10 seconds may be set. An application requiring a longer lock has to send repeating lock commands to maintain the lock.

| Request |  |
| --- | --- |
| CMD | U2FHID_LOCK |
| BCNT | 1 |
| DATA | Lock time in seconds 0..10. A value of 0 immediately releases the lock |

| Response at success |  |
| --- | --- |
| CMD | U2FHID_LOCK |
| BCNT | 0 |
| DATA | N/A |

## Vendor specific commands

A U2FHID may implement additional vendor specific commands that are not defined in this specification, yet being U2FHID compliant. Such commands, if implemented must have a command in the range between U2FHID_VENDOR_FIRST and U2FHID_VENDOR_LAST.